MAICEN 1125 — Module 6, Units 1 & 2

Parametric Building Design

Using Grasshopper and Rhino.Inside Revit

Assignment Report

| Group: | Group 5 |
| --- | --- |
| Course: | Zigurat Institute of Technology |
| Submission Date: | 26 April 2026 |
| Total Workload: | 20 hours (approx. 4 hours per student) |

# 1. Introduction

This report documents the development of a parametric building design script created using Grasshopper with the Rhino.Inside Revit plugin. The objective of this assignment was to shift from static manual modelling to a dynamic, adaptive parametric approach, enabling the generation of diverse architectural building solutions driven by a set of input parameters.

The script was developed collaboratively by Group 5 as part of Module 6 (Units 1 and 2) of the MAICEN 1125 course at Zigurat Institute of Technology. The final deliverable is a single Grasshopper definition capable of producing complete BIM elements directly in Autodesk Revit, including Levels, Floors, Columns, and a glass Facade.

## 1.1 Assignment Objectives

- Develop a parametric Grasshopper script driven by variable inputs

- Generate BIM-compliant building elements in Autodesk Revit using Rhino.Inside Revit

- Enforce design rules: plot coverage below 60%, minimum setback of 2.4 m, 5 to 50 floors, floor-to-floor height between 3.0 m and 5.0 m

- Output a data summary including floor areas, column count, and facade area

- Demonstrate parametric flexibility by producing multiple distinct building configurations

## 1.2 Tools and Software

| Parametric Modelling | Grasshopper 3D (within Rhino 7/8) |
| --- | --- |
| BIM Platform | Autodesk Revit 2024 |
| Integration Plugin | Rhino.Inside Revit |
| AI Assistance (Layout) | Raven AI for Grasshopper (node organization) |
| Version Control | GitHub — MAICEN-1125-M6-Grasshopper-Group5 |

# 2. Problem Statement

The assignment required the design of a multi-storey building on a rectangular plot with an area between 24,000 m² and 60,000 m². The building must satisfy the following mandatory design rules:

| Design Rule | Constraint |
| --- | --- |
| Plot area | Between 24,000 m² and 60,000 m² (reference values) |
| Building footprint | Less than 60% of the total plot area |
| Setback from plot boundary | Minimum 2.4 m (variable) |
| Number of floors | Between 5 and 50 floors |
| Floor-to-floor height | Between 3.0 m and 5.0 m |

# 3. Methodology and Script Development

The Grasshopper script was structured following a clear data flow from inputs on the left of the canvas to Revit outputs on the right. The team divided the work into logical modules, each responsible for one category of BIM element, allowing parallel development and easy debugging.

## 3.1 Input Parameters

All design variables are controlled through Grasshopper sliders and a polyline input, making the script fully parametric. Changing any slider immediately updates the entire building model in Revit.

| Parameter | Min Value | Max Value | Description |
| --- | --- | --- | --- |
| num_floors | 5 | 50 | Number of building floors |
| floor_height | 3.0 m | 5.0 m | Floor-to-floor height in metres |
| setback | 2.4 m | Variable | Distance from plot boundary |
| col_spacing | 4 m | Variable | Spacing between structural columns |
| plot_boundary | Polyline | — | Defines the outer plot perimeter |

## 3.2 Script Architecture

The script follows a left-to-right logical flow organized into five functional groups:

- Inputs Group — sliders, plot boundary polyline, and material swatches

- Levels Group — computes floor elevations and creates Revit Level elements

- Floors Group — generates floor boundary curves per level and creates Revit Floor elements

- Columns Group — calculates grid points within the footprint and creates Revit Column elements

- Facade Group — extrudes the building perimeter to full height and creates a glass DirectShape in Revit

## 3.3 Key Computational Logic

Total Building Height

The total height is computed by multiplying num_floors by floor_height. This single value drives both the Level elevations and the Facade extrusion height, ensuring consistency across all elements.

Footprint and Setback

The plot boundary polyline is offset inward by the setback value using the Inward Offset component. The resulting smaller curve defines the building footprint. The area of this footprint is checked against 60% of the plot area to enforce the coverage rule.

Floor Elevations

A Series component generates a list of elevation values starting at 0, incrementing by floor_height, for a count of num_floors. These values feed into the Floor Elevations component which produces level names and elevations, passed to Add Level in Revit.

Column Grid

Column base points are distributed across the building footprint using a spacing grid derived from the col_spacing parameter. The Spacing Calc component divides the footprint dimensions by the spacing value to determine the number of column rows and columns. Column lines are then created from base points to top points and passed to Add Column in Revit.

Facade Generation

The Facade component generates the perimeter wall lines (L output) of the building at each floor level. These lines are extruded vertically using the Extrude component with a Unit Z vector scaled by Amplitude to the total building height. The resulting Brep surfaces are sent to Add DirectShape in Revit with the Generic Models category and a Glass material applied.

# 4. BIM Outputs in Revit

The Grasshopper script produces four categories of BIM elements directly in Autodesk Revit via the Rhino.Inside Revit plugin. Each element is associated with the correct level and material.

| BIM Element | Revit Component | Description |
| --- | --- | --- |
| Levels | Add Level | One level per floor, named and positioned at the correct elevation |
| Floor Slabs | Add Floor (Boundary) | One floor element per level, using the inward-offset footprint boundary |
| Structural Columns | Add Column | Grid of columns from base to top of each floor, using Steel material |
| Facade | Add DirectShape | Glass curtain wall surface extruded to full building height, Generic Models category |

## 4.1 Data Summary

The script computes and displays the following summary data in Grasshopper output panels:

| Summary Output | Calculation Method |
| --- | --- |
| Total number of floors | num_floors slider value — direct output |
| Total floor area (m²) | Floor slab Breps → Area component → Mass Addition (sum) |
| Total number of columns | Column base points list → List Length component |
| Total facade area (m²) | Extruded facade Breps → Area component → Mass Addition (sum) |
| Plot area (m²) | Plot boundary curve → Area component |
| Footprint coverage (%) | Building footprint area divided by plot area × 100 |

# 5. Results — Different Building Configurations

The parametric nature of the script was tested by running three distinct configurations, each representing a different architectural scenario. The screenshots below demonstrate the script's ability to generate varied building solutions from the same definition.

## 5.1 Configuration 1 — Low-Rise Building

| num_floors | 5 |
| --- | --- |
| floor_height | 3.0 m |
| setback | 2.4 m |
| Total Height | 15.0 m |
| Column Spacing | 6 m |

This configuration represents a low-rise commercial building with minimal floor-to-floor height and maximum ground coverage within the 60% rule. The compact footprint and low profile suit a dense urban environment.

## 5.2 Configuration 2 — Mid-Rise Building

| num_floors | 20 |
| --- | --- |
| floor_height | 4.0 m |
| setback | 5.0 m |
| Total Height | 80.0 m |
| Column Spacing | 6 m |

The mid-rise configuration increases both the number of floors and the floor height, producing a taller building with a larger setback. The glass facade wraps the full height, demonstrating the automatic scaling of all BIM elements when sliders are adjusted.

## 5.3 Configuration 3 — High-Rise Building

| num_floors | 40 |
| --- | --- |
| floor_height | 5.0 m |
| setback | 8.0 m |
| Total Height | 200.0 m |
| Column Spacing | 6 m |

The high-rise configuration pushes the parameters towards the upper limits defined by the assignment brief. With 40 floors at 5.0 m each, the building reaches 200 m in height. The increased setback reduces the footprint further, keeping coverage well within the 60% threshold.

# 6. Design Rules Validation

The script enforces all assignment design rules automatically. The following table confirms compliance for each configuration tested:

| Design Rule | Config 1 | Config 2 | Config 3 |
| --- | --- | --- | --- |
| Floors between 5 and 50 | 5 ✓ | 20 ✓ | 40 ✓ |
| Floor height 3.0–5.0 m | 3.0 m ✓ | 4.0 m ✓ | 5.0 m ✓ |
| Setback ≥ 2.4 m | 2.4 m ✓ | 5.0 m ✓ | 8.0 m ✓ |
| Coverage < 60% | < 60% ✓ | < 60% ✓ | < 60% ✓ |

# 7. Development Workflow

The team followed an iterative development process, dividing responsibilities and integrating each module into a single cohesive Grasshopper definition.

## 7.1 Team Responsibilities

| Module | Responsibility |
| --- | --- |
| Inputs & Levels | Definition of all input parameters, plot boundary handling, floor elevation computation, Add Level to Revit |
| Floors | Floor boundary generation per level, Move to Floors logic, Add Floor to Revit with correct level association |
| Columns | Column grid computation, spacing calculation, column lines generation, Add Column to Revit |
| Facade | Facade perimeter lines, extrusion chain, DirectShape with Glass material, Add DirectShape to Revit |
| Summary & Documentation | Data summary panels, area calculations, coverage check, report writing and GitHub repository |

## 7.2 Version Control

The project repository is hosted on GitHub under the MAICEN organization. All files are organized in the following folder structure:

- Grasshopper/ — the main .gh script file

- Revit/ — the Revit project model file (.rvt)

- Rhino/ — the Rhino 3D model file (.3dm)

- Screenshots/ — Revit viewport screenshots for each configuration

- Results/ — data summary outputs and analysis

- Documentation/ — this report and supporting documents

# 8. Conclusion

This assignment successfully demonstrated the transition from static manual modelling to a fully parametric, rule-based approach to building design. The Grasshopper script developed by Group 5 is capable of generating complete multi-storey building solutions in Autodesk Revit from a small set of variable inputs.

All four required BIM elements — Levels, Floors, Columns, and Facade — are produced automatically and associated correctly with their respective levels in Revit. The script enforces all design rules including plot coverage, setback distance, and floor count and height ranges.

The parametric nature of the script makes it suitable for integration into multi-objective optimization workflows, where input parameters could be varied systematically to evaluate a large number of building configurations against performance criteria such as structural efficiency, daylighting, or energy use.

## 8.1 Key Achievements

- Fully parametric Grasshopper definition with 5 variable inputs

- 4 BIM element types generated directly in Revit

- Automatic enforcement of all 4 design rules

- Data summary panel with 6 computed outputs

- 3 distinct building configurations tested and verified

- Complete GitHub repository with version-controlled deliverables

## 8.2 Limitations and Future Work

- The facade is currently a single extruded surface. Future work could subdivide it into individual curtain wall panels with opening ratios.

- A roof slab element could be added on the topmost level to complete the building envelope.

- The script could be extended to support non-rectangular plot shapes using more complex offset algorithms.

- Integration with energy analysis tools such as Ladybug Tools would allow performance-based optimization of the building parameters.

Group 5 — MAICEN 1125 M6U1&U2 — Zigurat Institute of Technology — April 2026